HK-J260自上料搅拌车参数

| 型号规格 MODEL NO | 型号：HK-J260 Model: HK-J260 |  |
| --- | --- | --- |
| 型式性能 TYPE | 双向驾驶,铰接移动式、自装卸料、搅拌、运送 Two-way driving, articulated mobile, self loading, mixing, transporting |  |
| 功能 FUNCTION | 混凝土、沙浆、化肥复合、饲料等散状物料的湿式（或干式）拌合并能将物料自行卸到目的地 concrete, sand, fertilizer compound, wet or dry feed materias mixing and discharging |  |
| 生产功能：PRODUCTIVITY | 批次≤15分钟，每小时4批（8方） BATCH≤15MINS, 4BATCHS/HOUR |  |
| 驱动 | 4 x 4 |  |
| 转向 steering | 铰接转向 Articulated steering |  |
| 发动机： ENGINE | 云内 4缸2200转/分钟，78KW.中冷增压 YUNEI ENGINE: FOUR-CYLINDER 2200RPM,78KW |  |
| 行走速度：SPEED | 高速38.5公里/时 低速7.5公里/时 HIGH SPEED: 38.5KM/H; LOW SPEE:7.5KM/H |  |
| 档位 Gear number | 前进挡3个，倒挡3个 Three forwards, three backwards |  |
| 油耗 Fuel consumption | 怠速状态2L,工作状态5L-7L. Idle state 2L, working state 5L-7L. |  |
| 轮胎：TIRE | 16/70－20（钢丝轮胎） (Steel wire tyre) |  |
| 搅拌罐：DRUM | 几何容积2.6立方米，最佳搅拌1.8-2立方米 Geometric volume 2.6 cubic meters, the best stirring 1.8-2 cubic meters |  |
| 罐内螺旋叶片 Spiral blade in tank | 双螺旋线搅拌，配导流螺旋，使搅拌物料更均匀。左旋进料右旋出料 Double helix mixing, with the guide screw, so that the mixing material more uniform.Turn the feed left, turn the discharge right |  |
| 搅拌罐180度卸料 mixing tank 180°discharges | 罐体水平旋转达到180度。车辆周围卸料无死角The tank rotates horizontally to 180 degrees.There is no dead Angle for discharging around the vehicle |  |
| 溜料槽：TIPPING CHUTE | 配2节副溜料槽可拆卸，为增加卸料长度 Equipped with 2 chute for detachable to increase discharging length |  |
| 搅拌罐倾斜度 Tilt of mixing tank | 16度/16° |  |
| 铲斗：SHOVEL | 容量0.52立方 shovel capacity: 0.52³ |  |
| 铲斗放料开关 Bucket discharge switch | 液压闸门，电控 Hydraulic gate, electric control |  |
| 操纵杆：OPERATION HANDLE | 电控按钮四向先导手柄 ELECTRIC CONTROL BUTTON FOUR-WAY GUIDE HANDLE |  |
| 行走液压： MOVING HYDRAULIC | 泵（DENGHONG）PUMP:DENGHONG |  |
|  | 280变矩器（HANK） 280 Torque converter（HANK） |  |
| 工作液压 Work hydraulic | 泵/阀（DENGHONG） PUMP/valve: DENGHONG |  |
| 搅拌系统 Mixing system | 减速机（HANK） SPEED REDUCER:（HANK） |  |
|  | 搅拌马达(镇江大力) Mixing motor（ZHENJIANG DALI） |  |
| 驱动车桥Drive axle | 柳峰 LIUFENG |  |
| 供水系统： WATER SUPPLY | 上水：液压驱动自吸式水泵。 HYDRAULIC DRIVING SELF-PRIMIING PUMPS. |  |
| 高压洗车 HIGH-PRESSURE WATER PUMP WASH | 电子高压水泵冲洗 HIGH-PRESSURE WATER PUMP WASH |  |
| 称重系统 Weighing system | 电子称重系统（语言可选） Electronic weighing system (language optional） |  |
| 电压：VOLTAGE | 配电池两块24V,100A。 Battery 24V-100A 2 pieces |  |
| 液压油箱：HYDRAULIC OIL BOX | 容量110升夏季68号液压油，冬季用46号液压油，严寒地区配35号液压油。配备液位计。 TANKAGE110L, SUMMER:NO68 HYDRAULIC OIL, WINTER:NO46 HYDRAULIC OIL, VERY COLD ZONE: NO35 HYDRAULIC OIL |  |
| 水箱容积 Water tank capacity | 400L |  |
| 柴油箱容积 Diesel tank capacity | 110L |  |
| 质量 whole weight | 整备质量 6700KG COMPLETE VEHICLE KERB MASS 6700kg |  |
| 驾驶室 CAB | 防落物驾驶室 Prevent falling objects cab |  |
|  | 可调方向盘 Adjustable steering wheel |  |
|  | 可调式座椅 Adjustable seat |  |
|  | 安全带 Seat belts |  |
|  | 灭火器 Fire extinguisher |  |
|  | 倒车影像 Astern image |  |
| 车顶警灯ALARM LAMP | 标配 STANDARD CONFIGURATION |  |
| 收音机RADIO | 标配 STANDARD CONFIGURATION |  |
| 驾驶室空调 A/C | 可选optional |  |
| 逃生锤ESCAPE HAMMER | 标配STANDARD CONFIGURATION |  |
| 倒车声音提示 Astern sound prompt | 标配STANDARD CONFIGURATION |  |
| 行走控制方式 Walking control mode | 标配：机械 | 可选：电控 |
|  | STANDARD CONFIGURATION：mechanical | Optional：Electric control |

Overall size drawing：

Optional:

1：Engine brand.

2: A/C

3: Electronic weighing system

4: High pressure car washing machine

5: Electric control walking system

6: Mixing tank 270°discharges

7: Centralized lubrication system

8: Cement storage hopper